PK-88 Parameters

This is a list of all the parameters (DISP Z) that I have tested, totally successfully, with a PK88 and WinPack. Most of them won't have any affect on the performance of WinPack, but I haven't got time to sit and filter out the non-critical ones.

**** PLEASE READ AND TAKE NOTICE OF THE NEXT BIT!!!! ***** ***** NB NB NB ***** You MUST MUST MUST set JP3 correctly in the TNC to get the PK88 to work with WinPack. It is a 3 pin header, you MUST put the jumper on the two pins nearest the front of the TNC - the default is that it is on the two pins nearest the back. The setting for WinPack will also be correct for Packet, TPK, and most other modern packet software.

Once you've set the jumper, the default settings in WinPack Comms Setup for the connected and disconnected strings, and for "DCD shows con" (Yes) will be correct.

The default PK88 settings for AWLEN and PARITY are 7 and 3. To initially get WinPack to talk to the PK88, you may need to set WinPack to 7 databits, even parity. Once you have established communication, send the commands AWLEN 8, PARITY 0 and RESTART to the TNC - the screen will show some rubbish. Now set WinPack back to 8 databits, even parity.

See "TNC/Computer" on my TNCs page for an easier way. - IPL

Regard these settings as a good starting point - they work! Add things like BTEXT, set PACLEN and MAXFRAME to suit local conditions, etc.

Have fun! 73 - G4IDE.

Unproto   (URCall)

3Rdparty  OFF

8Bitconv  ON

ACKprior  OFF

ACRDisp   0

ACRPack   ON

AFilter   OFF

ALFDisp   ON

ALFPack   OFF

AUTOBaud  OFF

AWlen     8

Ax25l2v2  ON

AXDelay   0

AXHang    0

BBSmsgs   OFF

Beacon    EVERY 0

BKondel   ON

BText

CANline   $18

CANPac    $19

CASedisp  0

CBell     OFF

CFrom     all

CHCall    OFF

CHDouble  OFF

CHeck     30

CHSwitch  $00

CLKadj    0

CMdtime   10

CMSg      ON

COMmand   $03

Commode   CONVERSE

CONPerm   OFF

CONStamp  OFF

CPactime  OFF

CText     Send /HELP for help

CUstom    $0A15

DAYStamp  OFF

DELete    OFF

DFrom     all

DWait     16

Echo      OFF

EScape    OFF

Flow      ON

FRack     5

FREe      19040

FUlldup   OFF

HBaud     1200

HEAderln  ON

HId       OFF

HOMebbs   none

ILfpack   ON

KILONFWD  ON

LAstmsg   0

LIte      OFF

MAildrop  OFF

MAXframe  2

MBEll     OFF

MBx       none

MCon 0    (none)

MDigi     OFF

MDMon     OFF

MDPrompt  ^Z (CTRL-Z) or /EX to end

MFIlter   $00

MFrom     all

MId       0

MMsg      OFF

Monitor   4

MProto    OFF

MRpt      ON

MStamp    OFF

MTExt     PK-88 maildrop.

MTo       none

MXmit     OFF

MYAlias   none

MYcall    Your callsign!

MYMail    none

NEwmode   ON

NOmode    OFF

NUCr      OFF

NULf      OFF

NULLs     0

PACLen    128

PACTime   AFTER 10

PARity    0

PASs      $16

PASSAll   OFF

PErsist   63

PPersist  ON

RAdio     1

REDispla  $12

RELink    OFF

RESptime  0

REtry     10

SEndpac   $0D

SLottime  30

SQuelch   OFF

STArt     $11

STOp      $13

TBaud     9600

TIme      $14

TOne      3

TRACe     OFF

TRFlow    OFF

TRIes     0

TXdelay   30

TXFlow    OFF

USers     1

WHYnot    OFF

XFlow     OFF

XMITOk    ON

XOff      $13

XON       $11